彩色影像處理

作業3：簡化的CNN 數值辨識

M10915045 施信宏

完成了可以隨意指定0~9之間兩類別的比較，若要更改數值，請在下圖地方更改，路徑改成存放位置的地方。其餘程式碼部分已有註解，有疑問再請詳看。

類別 : 2、9

隨機15個影像分類結果

類別 3、7

Code :

import numpy as np

import cv2

from sklearn.metrics import classification_report

import itertools

import matplotlib.pyplot as plt

from sklearn.metrics import confusion_matrix

import random

trainPath = 'C:\\Users\\User\\Desktop\\NTUST\\image_processing\\HW3 readme\\train1000\\'

testPath = 'C:\\Users\\User\\Desktop\\NTUST\\image_processing\\HW3 readme\\test1000\\'

kernel1 = np.array([(-1, -1, 1, -1, 0, 1, -1, 1, 1)]).reshape(3, 3)

#預設的filter1

kernel2 = np.random.random(size=(3, 3))

kernel2 = (np.array((kernel2 / np.sum(kernel2))) - (1/9)) * 20

#自訂的filter2

# num1 , num2 為可指定 0 ~9

num1 = 2

num2 = 9

#讀取檔案，只取train的部分即可完成作業需求

def load_image(num1 , num2):

train = []

test = []

num1 = num1 * 100

num2 = num2 * 100

for i in range(num1 , num1 + 100):

img = cv2.imread(trainPath + str(i + 1) + ".png", 0)

train.append(cv2.resize(img, (8, 8)))

for i in range(num2,num2 + 100):

img = cv2.imread(trainPath + str(i + 1) + ".png", 0)

train.append(cv2.resize(img, (8, 8)))

return train

#max pooling

def pooling(inputMap, poolSize=3, poolStride=2, mode='max'):

"""INPUTS:

inputMap - input array of the pooling layer

poolSize - X-size(equivalent to Y-size) of receptive field

poolStride - the stride size between successive pooling squares

OUTPUTS:

outputMap - output array of the pooling layer

Padding mode - 'edge'

"""

# inputMap sizes

in_row, in_col = np.shape(inputMap)

# outputMap sizes

out_row, out_col = int(np.floor(in_row / poolStride)), int(np.floor(in_col / poolStride))

row_remainder, col_remainder = np.mod(in_row, poolStride), np.mod(in_col, poolStride)

if row_remainder != 0:

out_row += 1

if col_remainder != 0:

out_col += 1

outputMap = np.zeros((out_row, out_col))

# padding

temp_map = np.lib.pad(inputMap, ((0, poolSize - row_remainder), (0, poolSize - col_remainder)), 'edge')

# max pooling

for r_idx in range(0, out_row):

for c_idx in range(0, out_col):

startX = c_idx * poolStride

startY = r_idx * poolStride

poolField = temp_map[startY:startY + poolSize, startX:startX + poolSize]

poolOut = np.max(poolField)

outputMap[r_idx, c_idx] = poolOut

# retrun outputMap

return outputMap

#第一層捲積及完成max-pooling

def conv_pool_layer1(img, kernel):

# Conv_and_Pooling

conv_img = cv2.filter2D(img, -1, kernel)

max_img = pooling(conv_img, 2)

return max_img

#第二層捲積及完成max-pooling

def conv_pool_layer2(img, kernel):

# Conv_and_Pooling

conv_img = cv2.filter2D(img[:, :, 0], -1, kernel) + cv2.filter2D(img[:, :, 1], -1, kernel)

max_img = pooling(conv_img, 2)

return max_img

def model_train(num1, num2,train,kernel1,kernel2):

Y = np.zeros(200)

Y[:100] += num1

Y[100:] += num2

X = []

# Y true label , X predict 的結果

#經過兩層conv,max_pooling後的結果加入X

for img in train:

img = np.array([conv_pool_layer1(img, kernel1), conv_pool_layer1(img, kernel2)]).transpose(1, 2, 0)

img = np.array([conv_pool_layer2(img, kernel1), conv_pool_layer2(img, kernel2)]).transpose(1, 2, 0)

img = list(img.reshape(-1))

#平坦化 2*2*2 -> 8

img.append(1)

# 常數項加1

X.append(np.array(img))

X = np.array(X)

try:

A = np.dot(np.linalg.inv(np.dot(X.T, X)), (np.dot(X.T, Y)))

except np.linalg.LinAlgError:

print("LinAlgError")

Yp = np.dot(X, A)

y_pred = np.where(np.abs(Yp - num1) > np.abs(Yp - num2), num2, num1)

#將算出來的結果進行分類

y_true = Y

cnf_matrix = confusion_matrix(y_true, y_pred)

acc = cnf_matrix[0][0] + cnf_matrix[1][1]

print(acc)

return y_true , y_pred

#畫confusion_matrix

def plot_confusion_matrix(cm, classes,

normalize=False,

title='Confusion matrix',

cmap=plt.cm.Blues):

"""

This function prints and plots the confusion matrix.

Normalization can be applied by setting `normalize=True`.

"""

if normalize:

cm = cm.astype('float') / cm.sum(axis=1)[:, np.newaxis]

print("Normalized confusion matrix")

else:

print('Confusion matrix, without normalization')

cm = cm.T

print(cm)

plt.imshow(cm, interpolation='nearest', cmap=cmap)

plt.title(title)

tick_marks = np.arange(len(classes))

plt.xticks(tick_marks, classes)

plt.yticks(tick_marks, classes)

fmt = '.2f' if normalize else 'd'

thresh = cm.max() / 2.

for i, j in itertools.product(range(cm.shape[0]), range(cm.shape[1])):

plt.text(j, i, format(cm[i, j], fmt),

horizontalalignment="center",

color="white" if cm[i, j] > thresh else "black")

plt.tight_layout()

#隨機找15張圖，看其預測結果

def plot_images_labels_prediction(labels, prediction, num1, num2):

# fig = plt.gcf()

# fig.set_size_inches(12, 14)

plt.figure()

for i in range(0, 15):

r = random.randint(0, 199)

png = num1 * 100 + r if r < 100 else (num2 - 1) * 100 + r

img = cv2.imread(testPath + str(png + 1) + ".png", 0)

ax = plt.subplot(3, 5, 1 + i)

ax.imshow(img, cmap='gray')

title = str(prediction[r]) + "(" + str(png) + ")"

if labels[r] == prediction[r]:

color = 'b'

else:

color = 'r'

ax.set_title(title, fontsize=10, color=color)

ax.set_xticks([])

ax.set_yticks([])

def generate_confusion_mat(y_true, y_pred):

target_names = [str(num1), str(num2)]

cnf_matrix = confusion_matrix(y_true, y_pred)

plot_confusion_matrix(cnf_matrix, classes=target_names, normalize=False,

title='confusion matrix')

plot_images_labels_prediction(y_true, y_pred, num1, num2)

plt.show()

if __name__ == '__main__':

train  = load_image(num1 , num2)

y_true, y_pred = model_train(num1,num2,train,kernel1,kernel2)

generate_confusion_mat(y_true, y_pred)